#include "LSM6DSLSensor.h"

#include "mbed.h"

#include <XNucleoIKS01A2.h>

#include <cstdint>

#include <cstring>

#define WAIT_TIME_MS 500

DigitalOut led1(LED1);

static XNucleoIKS01A2 *sensorBoard = XNucleoIKS01A2::instance(D14, D15, D4, D5);

static LSM6DSLSensor *iNemo = sensorBoard->acc_gyro;

int main()

{

printf("This is the bare metal blinky example running on Mbed OS %d.%d.%d.\n",

MBED_MAJOR_VERSION, MBED_MINOR_VERSION, MBED_PATCH_VERSION);

iNemo->enable_x();

iNemo->enable_g();

uint8_t id;

iNemo->read_id(&id);

printf("iNemo replied id: 0x%X\r\n", id);

while (true)

{

led1 = !led1;

thread_sleep_for(WAIT_TIME_MS);

}

}

//April2

#include "LSM6DSLSensor.h"

#include "mbed.h"

#include <XNucleoIKS01A2.h>

#include <cstdint>

#include <cstring>

#define WAIT_TIME_MS 100

DigitalOut led1(LED1);

static XNucleoIKS01A2 *sensorBoard = XNucleoIKS01A2::instance(D14, D15, D4, D5);

static LSM6DSLSensor *iNemo = sensorBoard->acc_gyro;

int main()

{

printf("This is the bare metal blinky example running on Mbed OS %d.%d.%d.\n",

MBED_MAJOR_VERSION, MBED_MINOR_VERSION, MBED_PATCH_VERSION);

iNemo->enable_x();

iNemo->enable_g();

uint8_t id;

iNemo->read_id(&id);

printf("iNemo replied id: 0x%X\r\n", id);

int32_t xValues[3];

int32_t gValues[3];

while (true)

{

// Read acceleration values in mg (milli-gravity)

iNemo->get_x_axes(xValues);

printf("Accelerations [mg]  x: %d\ty: %d\tz: %d\r\n", xValues[0], xValues[1], xValues[2]);

// Read angular velocity values in mdps (milli-degrees per second)

iNemo->get_g_axes(gValues);

printf("Angular rates [mdps] x: %d\ty: %d\tz: %d\r\n", gValues[0], gValues[1], gValues[2]);

led1 = !led1;

thread_sleep_for(WAIT_TIME_MS);

}

}

//April2-1

#include "LSM6DSLSensor.h"

#include "mbed.h"

#include <XNucleoIKS01A2.h>

#include <cstdint>

#include <cstring>

#define WAIT_TIME_MS 100

DigitalOut led1(LED1);

static XNucleoIKS01A2 *sensorBoard = XNucleoIKS01A2::instance(D14, D15, D4, D5);

static LSM6DSLSensor *iNemo = sensorBoard->acc_gyro;

int main()

{

printf("This is the bare metal blinky example running on Mbed OS %d.%d.%d.\n",

MBED_MAJOR_VERSION, MBED_MINOR_VERSION, MBED_PATCH_VERSION);

iNemo->enable_x();

iNemo->enable_g();

uint8_t id;

iNemo->read_id(&id);

printf("iNemo replied id: 0x%X\r\n", id);

float xSens;

float gSens;

// Get accelerometer sensitivity in g/LSB

iNemo->get_x_sensitivity(&xSens);

// Convert to µg/LSB (micro-g) for easier readability

printf("Sensitivity [µg/LSB]: %d\r\n", (int)(xSens * 1000.0));

// Get gyroscope sensitivity in dps/LSB

iNemo->get_g_sensitivity(&gSens);

// Convert to mdps/LSB (milli-dps) for easier readability

printf("Sensitivity [mdps/LSB]: %d\r\n", (int)(gSens * 1000.0));

int32_t xValues[3];

int32_t gValues[3];

while (true)

{

// Read acceleration values in mg (milli-gravity)

iNemo->get_x_axes(xValues);

printf("Accelerations [mg]  x: %d\ty: %d\tz: %d\r\n", xValues[0], xValues[1], xValues[2]);

// Read angular velocity values in mdps (milli-degrees per second)

iNemo->get_g_axes(gValues);

printf("Angular rates [mdps] x: %d\ty: %d\tz: %d\r\n", gValues[0], gValues[1], gValues[2]);

led1 = !led1;

thread_sleep_for(WAIT_TIME_MS);

}

}

//April2-2