#include "LSM6DSLSensor.h"

#include "mbed.h"

#include <XNucleoIKS01A2.h>

#include <cstdint>

#include "string.h"

// Sampling delay

#define WAIT_TIME_MS 5

// Union for transmitting float values as byte arrays

union MyFloat {

float value;

uint8_t byte[4];

} gSens, xSens;

// Raw sensor data buffers

int16_t xValues[3];

int16_t gValues[3];

// High-speed serial communication to MATLAB

UnbufferedSerial pc(USBTX, USBRX, 921600);

// Message buffer for formatted output

char msg[30];

// Built-in LED for visual activity indicator

DigitalOut led1(LED1);

// Initialize sensor board and IMU (accelerometer + gyroscope)

static XNucleoIKS01A2 *sensorBoard = XNucleoIKS01A2::instance(D14, D15, D4, D5);

static LSM6DSLSensor *iNemo = sensorBoard->acc_gyro;

/

* Read a 16-bit integer (2 bytes) from the serial port, byte by byte.

*/

int16_t readInt16FromSerial() {

uint8_t bytes[2];

while (pc.read(&bytes[0], 1) != 1); // wait for first byte

while (pc.read(&bytes[1], 1) != 1); // wait for second byte

int16_t value;

memcpy(&value, bytes, 2); // combine into int16_t

return value;

}

/

* Wait until receiving the start command character 's'.

*/

char waitForCommandStart() {

char cmd;

do {

pc.read(&cmd, 1);

} while (cmd != 's');

return cmd;

}

/

* Send the sensitivity values of accelerometer and gyroscope to MATLAB.

* Each float is sent as 4 separate bytes.

*/

void sendSensorSensitivities() {

iNemo->get_x_sensitivity(&xSens.value);

for (int i = 0; i < 4; i++)

pc.write(&xSens.byte[i], 1);

iNemo->get_g_sensitivity(&gSens.value);

for (int i = 0; i < 4; i++)

pc.write(&gSens.byte[i], 1);

}

/

* Read and send raw sensor data (acceleration and gyroscope).

* Repeat for nSamples times, with sampleperiod (in ms) between each.

*/

void sendSensorData(int16_t nSamples, int16_t sampleperiod) {

for (int i = 0; i < nSamples; i++) {

iNemo->get_x_axes_raw(xValues);      // 3 x int16_t = 6 bytes

pc.write(xValues, 6);

iNemo->get_g_axes_raw(gValues);      // 3 x int16_t = 6 bytes

pc.write(gValues, 6);

led1 = !led1;                         // Toggle LED for visual feedback

thread_sleep_for(sampleperiod - 5);  // Wait for next sample

}

}

/**

* Main function that handles all communication with MATLAB.

* Continuously waits for parameters and sends sensor data.

*/

int main() {

iNemo->enable_x();  // Enable accelerometer

iNemo->enable_g();  // Enable gyroscope

while (true) {

int16_t sampleperiod = readInt16FromSerial(); // e.g., 20 ms

int16_t nSamples = readInt16FromSerial();     // e.g., 1000

waitForCommandStart();                        // Wait for 's'

uint8_t id;

iNemo->read_id(&id);                          // Read sensor ID

sprintf(msg, "iNemo replied id: 0X%X.\r\n", id);

pc.write(msg, strlen(msg));                   // Send ID info

sendSensorSensitivities();                    // Send sensitivity factors

sendSensorData(nSamples, sampleperiod);       // Send actual sensor data

}

}

//9April