clear

close all

clc

%% 1. Setup Serial Communication

% Replace "COM3" with your actual COM port

s = serialport("COM5", 921600);

%configureTerminator(s, "");  % No line terminator for binary comm

%% 2. Define Parameters

sampleperiod = int16(20);   % Sample period in milliseconds

nSamples = int16(1000);     % Number of samples to read

% Send sample period and number of samples to the microcontroller

write(s, typecast(sampleperiod, 'uint8'), "uint8");

write(s, typecast(nSamples, 'uint8'), "uint8");

% Send the start character 's' to begin streaming

write(s, 's', "char");

%% 3. Wait for Sensor ID Confirmation (optional)

% Reads a line like: "iNemo replied id: 0X6A"

idLine = readline(s);

disp(idLine)

%% 4. Receive Sensitivity Factors (4 bytes each for float)

% Receive accelerometer sensitivity

xBytes = read(s, 4, "uint8");

xSens = typecast(uint8(xBytes), "single");

% Receive gyroscope sensitivity

gBytes = read(s, 4, "uint8");

gSens = typecast(uint8(gBytes), "single");

fprintf("Accel Sensitivity: %.3f   |   Gyro Sensitivity: %.3f\n", xSens, gSens);

%% 5. Initialize Data Buffers

xAcc = nan(1, nSamples);

yAcc = nan(1, nSamples);

zAcc = nan(1, nSamples);

xAng = nan(1, nSamples);

yAng = nan(1, nSamples);

zAng = nan(1, nSamples);

t = (0:(nSamples-1)) * double(sampleperiod);

%% 6. Setup Live Plots

figure("Name", "IMU Real-Time Data", "Color", "w");

subplot(2,3,1); xA = plot(t, xAcc); title("X Acceleration");

subplot(2,3,2); yA = plot(t, yAcc); title("Y Acceleration");

subplot(2,3,3); zA = plot(t, zAcc); title("Z Acceleration");

subplot(2,3,4); xW = plot(t, xAng); title("X Angular Velocity");

subplot(2,3,5); yW = plot(t, yAng); title("Y Angular Velocity");

subplot(2,3,6); zW = plot(t, zAng); title("Z Angular Velocity");

%% 7. Start Reading and Plotting Data

disp("Receiving and plotting data...")

tic

for kk = 1:nSamples

% Read 6 int16 values = 12 bytes = 6 elements

data = read(s, 6, "int16");

% Process acceleration (ADC code × sensitivity)

xAcc(kk) = double(data(1)) * xSens;

yAcc(kk) = double(data(2)) * xSens;

zAcc(kk) = double(data(3)) * xSens;

% Process gyro (ADC code × sensitivity ÷ 1000 to convert to dps)

xAng(kk) = double(data(4)) * gSens / 1000;

yAng(kk) = double(data(5)) * gSens / 1000;

zAng(kk) = double(data(6)) * gSens / 1000;

% Update plots live

xA.YData = xAcc;

yA.YData = yAcc;

zA.YData = zAcc;

xW.YData = xAng;

yW.YData = yAng;

zW.YData = zAng;

end

toc

%% 8. Cleanup

clear s

disp("Done.")