clear; clc; close all;

%% Serial setup

s = serialport("COM5", 115200);  % Change COM port if needed

nSamples = 100;

%% Read sensitivity (4 bytes = 1 float)

rawSens = read(s, 4, "uint8");

sensitivity = typecast(uint8(rawSens), 'single');

sensitivity = sensitivity / 1000;  % Convert from mg to g

fprintf("Received sensitivity: %.6f g/LSB\n", sensitivity);

%% Read 100 int16 raw Z-axis values

rawZ = read(s, nSamples, "int16");

%% Convert to acceleration in g

zAcc = double(rawZ) * sensitivity;

%% Time vector

t = (0:nSamples-1) * 0.01;

%% Plot

plot(t, zAcc, 'b.-');

xlabel("Time (s)");

ylabel("Z Acceleration (g)");

title("Z-Axis Acceleration");

%% Stats

fprintf("Mean: %.5f g\n", mean(zAcc));

fprintf("Std Dev: %.5f g\n", std(zAcc));

%% Optional: Save

save('zAccData_in_g.mat', 'zAcc');